ROI G. Biv

ROI Guided Boundary Identification and Validation

Complete Setup and Fine-Tuning Guide

Lenovo Legion Pro 5i Gen 8 ¦ Pop!_OS 22.04 LTS RTX 4060 8GB ¦ 16GB DDR5 ¦ Cellpose 3.x

For researchers with Linux/Python experience

SECTION 1 ¦ Overview and Architecture

# 1. Overview

This guide walks through the complete pipeline for fine-tuning a Cellpose model on two-photon calcium imaging data, from a bare-metal Linux install through iterative retraining and automated inference deployment. The workflow is designed around the Cellpose GUI for annotation correction, eliminating napari overhead while keeping the review loop fast and intuitive.

## 1.1  Pipeline Architecture

The end-to-end workflow consists of five stages:

| Stage | Description |
| --- | --- |
| 1. Environment | Pop!_OS install, NVIDIA driver, CUDA, conda env, Cellpose |
| 2. Data Preparation | Export ImageJ ROIs to Cellpose mask format, preprocessing pipeline |
| 3. Initial Training | Fine-tune cyto3 base model on 50-100 annotated image pairs |
| 4. Iterative Review | Cellpose GUI correction loop, incremental dataset growth |
| 5. Deployment | Batch inference script, trace extraction, confidence scoring |

## 1.2  Hardware Context

All commands and parameters in this guide are tuned for the Lenovo Legion Pro 5i Gen 8 running an RTX 4060 with 8GB VRAM. The critical constraint is VRAM: batch size must stay at 4 or below for 512x512 images to avoid OOM during training. Where the 4050 or 4070 variants require different parameters, this is noted explicitly.

NOTE  Training on this machine is fast enough for same-day iteration at 50-100 image pairs. A 200-image, 300-epoch run takes approximately 3.5-5.5 hours and is best left overnight.

SECTION 2 ¦ Operating System Setup

# 2. Pop!_OS Installation

## 2.1  Why Pop!_OS Over Plain Ubuntu

Pop!_OS 22.04 LTS is the recommended distribution for this hardware for three specific reasons:

- The NVIDIA ISO ships with proprietary NVIDIA drivers pre-installed and correctly configured, eliminating the most common source of broken GPU setups on dual-GPU laptops.

- The built-in hybrid graphics switcher (switchable via Settings > Power) correctly handles the Intel iGPU + RTX 4060 combo without manual prime-select juggling.

- The distribution is Ubuntu-based (22.04 LTS), so all apt packages, PPAs, and CUDA toolkits install identically to Ubuntu.

## 2.2  Download and Install

Download the NVIDIA ISO specifically. Do not use the Intel/AMD ISO.

# Download from: https://pop.system76.com # Select: NVIDIA version of Pop!_OS 22.04 LTS # Verify checksum (SHA256 provided on download page) sha256sum pop-os_22.04_amd64_nvidia_*.iso # Flash to USB (replace /dev/sdX with your drive) sudo dd if=pop-os_22.04_amd64_nvidia_*.iso of=/dev/sdX bs=4M status=progress sync

Boot from USB. During install:

- Select 'Clean Install' — this replaces Windows. Dual-boot is possible but adds complexity that is not worth it for a dedicated workstation.

- Enable full-disk encryption (LUKS) if your imaging data is sensitive. This adds no measurable performance overhead for training workloads.

- When prompted for hostname, set something memorable — you will SSH into this machine for overnight jobs.

## 2.3  Post-Install System Configuration

After first boot, apply all system updates before touching NVIDIA configuration:

sudo apt update && sudo apt full-upgrade -y sudo apt install -y build-essential git curl wget htop nvtop lm-sensors sudo reboot

## 2.4  NVIDIA Driver Verification

The Pop!_OS NVIDIA ISO should have already installed the correct driver. Verify:

nvidia-smi # Expected output: driver version 535.x or higher, CUDA Version: 12.x # RTX 4060 should be listed as device 0 # If driver is missing or wrong version: sudo apt install -y system76-driver-nvidia sudo reboot

## 2.5  GPU Mode for Training

The Legion Pro 5i uses NVIDIA PRIME hybrid graphics. For training, you must force dedicated GPU mode so PyTorch targets the RTX 4060 directly rather than routing through the Intel iGPU.

# Method 1: GUI - Settings > Power > Graphics Mode > NVIDIA Graphics # Requires logout/login # Method 2: CLI sudo system76-power graphics nvidia sudo reboot # Verify active GPU glxinfo ¦ grep 'OpenGL renderer' # Should show: GeForce RTX 4060 Laptop GPU/PCIe/SSE2

WARNING  Running training in 'Hybrid' or 'Compute' mode will cause PyTorch to either fail to find the GPU or run at significantly reduced throughput. Always switch to NVIDIA mode before a training run.

## 2.6  LenovoLegionLinux Fan Control (Optional but Recommended)

For sustained training runs, the default fan curve is conservative. The LenovoLegionLinux driver gives you control over fan behavior to prevent thermal throttling during long jobs.

# Install dependencies sudo apt install -y dkms linux-headers-$(uname -r) # Clone and install git clone https://github.com/johnfanv2/LenovoLegionLinux.git cd LenovoLegionLinux make sudo make install sudo modprobe lenovo_legion_laptop # For persistent loading: echo 'lenovo_legion_laptop' ¦ sudo tee -a /etc/modules # Set aggressive fan curve for training (0-255 scale) # Edit /etc/legion_linux.cfg after install for permanent settings

SECTION 3 ¦ Python Environment Setup

# 3. Conda Environment and Cellpose

## 3.1  Install Miniconda

wget https://repo.anaconda.com/miniconda/Miniconda3-latest-Linux-x86_64.sh bash Miniconda3-latest-Linux-x86_64.sh -b -p $HOME/miniconda3 eval "$($HOME/miniconda3/bin/conda shell.bash hook)" conda init bash source ~/.bashrc # Disable auto-activation of base (cleaner shell) conda config --set auto_activate_base false

## 3.2  Create the ROI G. Biv Environment

Pin PyTorch to CUDA 12.1 — this matches the driver installed by Pop!_OS and is the most stable combination for RTX 4060.

conda create -n roigbiv python=3.10 -y conda activate roigbiv # PyTorch with CUDA 12.1 pip install torch torchvision torchaudio --index-url https://download.pytorch.org/whl/cu121 # Verify GPU is visible python -c "import torch; print(torch.cuda.is_available()); print(torch.cuda.get_device_name(0))" # Expected: True / NVIDIA GeForce RTX 4060 Laptop GPU

## 3.3  Install Cellpose and Supporting Packages

# Cellpose with GUI support pip install cellpose[gui] # Image I/O and processing pip install tifffile scikit-image imageio # Scientific stack pip install numpy scipy pandas matplotlib # ROI handling (for ImageJ/Fiji ROI conversion) pip install roifile # Notebook support (optional, useful for exploration) pip install jupyter ipykernel python -m ipykernel install --user --name roigbiv --display-name 'ROI G. Biv' # Freeze environment for reproducibility pip freeze > ~/roigbiv_requirements.txt

## 3.4  Verify Cellpose GPU Access

python -c " from cellpose import models import torch print('CUDA available:', torch.cuda.is_available()) model = models.Cellpose(gpu=True, model_type='cyto3') print('Cellpose GPU model loaded successfully') "

## 3.5  Project Directory Structure

Establish a clean directory layout before starting. Consistent paths matter when iterating across multiple training runs.

mkdir -p ~/roigbiv/{data/{raw,annotated,masks},models/{checkpoints,deployed}, scripts,logs,inference/{output,traces}} # Layout: # ~/roigbiv/ # ├── data/ # │ ├── raw/ # original .tif stacks from microscope # │ ├── annotated/ # mean/max projections for training # │ └── masks/ # Cellpose-format uint16 label masks # ├── models/ # │ ├── checkpoints/ # model saved at each training run # │ └── deployed/ # current production model # ├── scripts/ # all Python scripts # ├── logs/ # training logs # └── inference/ # ├── output/ # raw mask output from batch inference # └── traces/ # extracted fluorescence traces

SECTION 4 ¦ Data Preparation

# 4. Preparing Training Data

## 4.1  Input Image Format

Cellpose expects 2D images (not stacks) for training. For two-photon calcium imaging, generate per-field-of-view mean and maximum intensity projections from your raw stacks. These serve as the two input channels — Cellpose handles multi-channel input natively.

## 4.2  Projection Generation Script

Save as ~/roigbiv/scripts/make_projections.py:

#!/usr/bin/env python3 """ Generate mean and max projections from raw two-photon .tif stacks. Outputs paired files: FOVNAME_mean.tif and FOVNAME_max.tif """ import tifffile import numpy as np from pathlib import Path import argparse def make_projections(raw_dir, out_dir, percentile_clip=(0.5, 99.5)): raw_dir = Path(raw_dir) out_dir = Path(out_dir) out_dir.mkdir(parents=True, exist_ok=True) for tif in sorted(raw_dir.glob('*.tif')): stack = tifffile.imread(tif).astype(np.float32) # Handle (T, Y, X) or (Z, Y, X) stacks if stack.ndim == 2: stack = stack[np.newaxis, ...] mean_proj = stack.mean(axis=0) max_proj = stack.max(axis=0) # Percentile clip and rescale to uint16 for proj, suffix in [(mean_proj, 'mean'), (max_proj, 'max')]: lo, hi = np.percentile(proj, percentile_clip) proj = np.clip((proj - lo) / (hi - lo + 1e-8), 0, 1) proj = (proj * 65535).astype(np.uint16) out_path = out_dir / f'{tif.stem}_{suffix}.tif' tifffile.imwrite(out_path, proj) print(f'Wrote {out_path}') if __name__ == '__main__': ap = argparse.ArgumentParser() ap.add_argument('raw_dir') ap.add_argument('out_dir') args = ap.parse_args() make_projections(args.raw_dir, args.out_dir)

# Usage conda activate roigbiv python ~/roigbiv/scripts/make_projections.py ~/roigbiv/data/raw ~/roigbiv/data/annotated

## 4.3  Converting ImageJ ROIs to Cellpose Masks

Cellpose requires uint16 label masks where each ROI is a filled region with a unique integer ID (1, 2, 3...). If your existing annotations are ImageJ .roi or .zip files, use the following conversion script.

Save as ~/roigbiv/scripts/convert_rois.py:

#!/usr/bin/env python3 """ Convert ImageJ ROI .zip files to Cellpose uint16 label masks. Expects: FOVNAME.zip alongside FOVNAME_mean.tif in the annotated directory. Outputs: FOVNAME_masks.tif in the masks directory. """ import roifile import tifffile import numpy as np from pathlib import Path from skimage.draw import polygon import argparse def roi_zip_to_mask(roi_zip_path, reference_tif_path, out_mask_path): ref = tifffile.imread(reference_tif_path) H, W = ref.shape[-2], ref.shape[-1] mask = np.zeros((H, W), dtype=np.uint16) rois = roifile.roiread(str(roi_zip_path)) for idx, roi in enumerate(rois, start=1): coords = roi.coordinates() if coords is None or len(coords) < 3: continue rr, cc = polygon(coords[:, 1], coords[:, 0], shape=(H, W)) mask[rr, cc] = idx tifffile.imwrite(out_mask_path, mask) print(f' {len(rois)} ROIs -> {out_mask_path}') def batch_convert(annotated_dir, masks_dir): annotated_dir = Path(annotated_dir) masks_dir = Path(masks_dir) masks_dir.mkdir(parents=True, exist_ok=True) for zip_file in sorted(annotated_dir.glob('*.zip')): stem = zip_file.stem ref_tif = annotated_dir / f'{stem}_mean.tif' if not ref_tif.exists(): print(f'WARNING: No mean projection found for {stem}, skipping') continue out_mask = masks_dir / f'{stem}_masks.tif' print(f'Converting {zip_file.name} ...') roi_zip_to_mask(zip_file, ref_tif, out_mask) if __name__ == '__main__': ap = argparse.ArgumentParser() ap.add_argument('annotated_dir') ap.add_argument('masks_dir') args = ap.parse_args() batch_convert(args.annotated_dir, args.masks_dir)

# Usage python ~/roigbiv/scripts/convert_rois.py ~/roigbiv/data/annotated ~/roigbiv/data/masks

## 4.4  Cellpose Training File Naming Convention

Cellpose's file discovery requires specific naming. Images and their masks must share the same stem, with the mask file appended with _seg or specified via the masks argument. The simplest convention is to keep them in separate directories and pass both paths explicitly to the trainer.

Final expected layout before training:

~/roigbiv/data/annotated/ FOV001_mean.tif FOV001_max.tif FOV002_mean.tif FOV002_max.tif ... ~/roigbiv/data/masks/ FOV001_masks.tif # uint16, 0=background, 1..N=ROI IDs FOV002_masks.tif ...

NOTE  Cellpose training uses the mean projection as the primary channel. Optionally, the max projection can be passed as a second channel. Start single-channel for simplicity; add the second channel during retraining if the model struggles with cell boundaries.

## 4.5  Dataset Validation Script

Always validate your dataset before training to catch shape mismatches and empty masks early.

#!/usr/bin/env python3 # ~/roigbiv/scripts/validate_dataset.py import tifffile, numpy as np from pathlib import Path annotated = Path('~/roigbiv/data/annotated').expanduser() masks_dir = Path('~/roigbiv/data/masks').expanduser() issues = [] pairs = 0 for mask_path in sorted(masks_dir.glob('*_masks.tif')): stem = mask_path.stem.replace('_masks', '') img_path = annotated / f'{stem}_mean.tif' if not img_path.exists(): issues.append(f'MISSING image: {stem}_mean.tif') continue img = tifffile.imread(img_path) mask = tifffile.imread(mask_path) if img.shape[-2:] != mask.shape[-2:]: issues.append(f'SHAPE MISMATCH: {stem} img={img.shape} mask={mask.shape}') if mask.max() == 0: issues.append(f'EMPTY MASK: {stem}') else: pairs += 1 print(f'OK {stem}: {mask.max()} ROIs, image dtype={img.dtype}') print(f'\n{pairs} valid pairs found.') if issues: print('\nISSUES:') for i in issues: print(' ', i) else: print('All pairs validated successfully.')

SECTION 5 ¦ Training

# 5. Fine-Tuning the Cellpose Model

## 5.1  Training Parameters for RTX 4060

The following parameters are calibrated for 8GB VRAM and 512x512 two-photon images. The most critical parameter is batch_size — at 8GB VRAM, batch size 4 is the safe ceiling for the full Cellpose UNet loaded with CUDA overhead.

| Parameter | Recommended Value | Notes |
| --- | --- | --- |
| model_type | cyto3 | Best pretrained base for calcium imaging |
| batch_size | 4 | Safe for 8GB VRAM at 512x512. Use 2 for 1024x1024. |
| n_epochs | 100-300 | 100 for initial run; increase with dataset size |
| learning_rate | 0.1 | Cellpose default; reduce to 0.05 if training loss oscillates |
| weight_decay | 1e-5 | Regularization; keep at default |
| min_train_masks | 5 | Minimum ROIs per image; exclude sparse fields |
| rescale | True | Normalize cell size across FOVs |
| diameter | 0 | Auto-estimate from masks; override if cells are uniform size |

## 5.2  Training Script (Run 1 — Initial)

Save as ~/roigbiv/scripts/train.py:

#!/usr/bin/env python3 """ ROI G. Biv — Cellpose fine-tuning script. Usage: python train.py --run_id run001 [--epochs 100] [--lr 0.1] """ import argparse, logging, sys, time from pathlib import Path import numpy as np import tifffile from cellpose import models, io, train import torch # ── Config ──────────────────────────────────────────────────────────── BASE_DIR = Path.home() / 'roigbiv' DATA_DIR = BASE_DIR / 'data' / 'annotated' MASKS_DIR = BASE_DIR / 'data' / 'masks' MODELS_DIR = BASE_DIR / 'models' / 'checkpoints' LOGS_DIR = BASE_DIR / 'logs' MODELS_DIR.mkdir(parents=True, exist_ok=True) LOGS_DIR.mkdir(parents=True, exist_ok=True) def load_dataset(data_dir, masks_dir): """Load images and masks into lists for Cellpose trainer.""" images, masks = [], [] mask_files = sorted(masks_dir.glob('*_masks.tif')) for mf in mask_files: stem = mf.stem.replace('_masks', '') img_path = data_dir / f'{stem}_mean.tif' if not img_path.exists(): print(f'WARNING: Missing {img_path}, skipping') continue img = tifffile.imread(img_path).astype(np.float32) mask = tifffile.imread(mf) if mask.max() == 0: print(f'WARNING: Empty mask {mf.name}, skipping') continue images.append(img) masks.append(mask) return images, masks def main(): ap = argparse.ArgumentParser() ap.add_argument('--run_id', required=True) ap.add_argument('--epochs', type=int, default=100) ap.add_argument('--lr', type=float, default=0.1) ap.add_argument('--batch_size', type=int, default=4) ap.add_argument('--base_model', default='cyto3', help='Path to previous checkpoint or Cellpose model name') args = ap.parse_args() log_path = LOGS_DIR / f'{args.run_id}.log' logging.basicConfig(level=logging.INFO, format='%(asctime)s %(levelname)s %(message)s', handlers=[logging.FileHandler(log_path), logging.StreamHandler()]) log = logging.getLogger('roigbiv') log.info(f'PyTorch CUDA: {torch.cuda.is_available()}') log.info(f'GPU: {torch.cuda.get_device_name(0) if torch.cuda.is_available() else "CPU"}') log.info(f'Run ID: {args.run_id}, base model: {args.base_model}') # Load dataset images, masks = load_dataset(DATA_DIR, MASKS_DIR) log.info(f'Loaded {len(images)} image/mask pairs') assert len(images) >= 3, 'Need at least 3 pairs for training' # Train/val split (90/10) n_val = max(1, int(len(images) * 0.1)) train_imgs, val_imgs = images[n_val:], images[:n_val] train_masks, val_masks = masks[n_val:], masks[:n_val] log.info(f'Train: {len(train_imgs)}, Val: {len(val_imgs)}') # Initialize model model = models.CellposeModel(gpu=True, model_type=args.base_model) # Train out_model_path = MODELS_DIR / args.run_id t0 = time.time() model_path = train.train_seg( model.net, train_data=train_imgs, train_labels=train_masks, test_data=val_imgs, test_labels=val_masks, channels=[0, 0], # single-channel: mean projection normalize=True, learning_rate=args.lr, weight_decay=1e-5, n_epochs=args.epochs, batch_size=args.batch_size, min_train_masks=5, save_path=str(MODELS_DIR), save_every=50, model_name=args.run_id, ) elapsed = time.time() - t0 log.info(f'Training complete in {elapsed/60:.1f} min. Model: {model_path}') if __name__ == '__main__': main()

## 5.3  Running the First Training Job

conda activate roigbiv # Validate dataset before committing to a run python ~/roigbiv/scripts/validate_dataset.py # Run initial training (100 epochs, safe for 50-100 image pairs) python ~/roigbiv/scripts/train.py --run_id run001 --epochs 100 # For an overnight run with a larger dataset: nohup python ~/roigbiv/scripts/train.py --run_id run001 --epochs 300 \ > ~/roigbiv/logs/run001_nohup.out 2>&1 & echo $! > ~/roigbiv/logs/run001.pid # Monitor progress tail -f ~/roigbiv/logs/run001.log # Check GPU utilization during training nvtop # or: watch -n1 nvidia-smi

## 5.4  Expected Training Times (RTX 4060, batch size 4)

| Dataset Size | 100 epochs | 300 epochs |
| --- | --- | --- |
| 50 image pairs | 18-28 min | 55-85 min |
| 100 image pairs | 35-55 min | 1.75-2.75 hrs |
| 200 image pairs | 70-110 min | 3.5-5.5 hrs |

NOTE  If GPU utilization (visible in nvtop) drops below 70% during training, the CPU is the bottleneck — the DataLoader is not feeding batches fast enough. Add --num_workers 4 if the Cellpose API exposes it, or reduce image resolution preprocessing overhead.

SECTION 6 ¦ Iterative Review and Retraining

# 6. The Cellpose GUI Review Loop

## 6.1  Launching the Cellpose GUI

The Cellpose GUI provides a complete annotation correction environment. It displays your raw image, overlays the predicted masks, and allows you to add, delete, and reshape ROIs interactively. Corrected outputs save directly in Cellpose mask format, ready for the next training run.

# Always launch with GPU flag so inference during review is fast conda activate roigbiv python -m cellpose --gpu

## 6.2  GUI Workflow — Step by Step

- Open image: File > Load Image. Load the mean projection .tif for the FOV you want to review.

- Load your fine-tuned model: Model > Add Custom Torch Model. Navigate to ~/roigbiv/models/checkpoints/run001.

- Set Channels: Chan 1 = 0 (grayscale), Chan 2 = 0. This matches your training configuration.

- Run segmentation: Click 'run segmentation'. The model will generate an initial mask overlay.

- Correct ROIs using the toolbar: Draw new ROIs (D), delete incorrect ones (right-click), merge over-split cells (M). See key bindings below.

- Save: File > Save Masks. This writes a *_seg.npy file containing your corrected masks alongside the image. This file is what gets fed into the next training run.

## 6.3  Key Bindings

| Key / Action | Function |
| --- | --- |
| D (draw) | Draw a new ROI by clicking boundary points |
| Right-click ROI | Delete the selected ROI |
| M (merge) | Merge two adjacent over-split ROIs |
| Ctrl+Z | Undo last action |
| Scroll wheel | Zoom in/out |
| Middle-click drag | Pan |
| Ctrl+S | Save masks |

## 6.4  Converting GUI Output Back to Training Format

The GUI saves *_seg.npy files. Extract the masks and add them to your training dataset with this script.

Save as ~/roigbiv/scripts/ingest_corrections.py:

#!/usr/bin/env python3 """ Ingest Cellpose GUI corrections (*_seg.npy) into the training dataset. Run after each review session to update masks for retraining. """ import numpy as np import tifffile from pathlib import Path import shutil GUI_DIR = Path('~/roigbiv/data/annotated').expanduser() MASKS_DIR = Path('~/roigbiv/data/masks').expanduser() updated = 0 for seg_file in sorted(GUI_DIR.glob('*_seg.npy')): data = np.load(seg_file, allow_pickle=True).item() masks = data.get('masks', None) if masks is None or masks.max() == 0: print(f'WARNING: No masks in {seg_file.name}, skipping') continue stem = seg_file.stem.replace('_mean_seg', '').replace('_seg', '') out_path = MASKS_DIR / f'{stem}_masks.tif' tifffile.imwrite(out_path, masks.astype(np.uint16)) print(f'Updated: {stem} ({masks.max()} ROIs) -> {out_path.name}') updated += 1 print(f'\n{updated} mask files updated.')

# Run after each GUI session python ~/roigbiv/scripts/ingest_corrections.py

## 6.5  Retraining with an Updated Dataset

Each retraining run should build on the previous model rather than starting from scratch. Pass the checkpoint from the last run as the base model:

# Retrain from previous checkpoint python ~/roigbiv/scripts/train.py \ --run_id run002 \ --epochs 150 \ --base_model ~/roigbiv/models/checkpoints/run001 # After validating run002 performance, promote to deployed: cp ~/roigbiv/models/checkpoints/run002 ~/roigbiv/models/deployed/current_model

## 6.6  Iteration Strategy

Follow this cadence for building model trust incrementally:

| Run | Strategy |
| --- | --- |
| Run 1 (50 FOVs) | Train 100 epochs from cyto3. Correct the worst 20% of predictions in the GUI. |
| Run 2 (70 FOVs) | Retrain 150 epochs from run001. Review all new FOVs plus 10 re-reviews. |
| Run 3 (100+ FOVs) | Retrain 200 epochs. Spot-check random 10% of predictions. Convergence should be visible. |
| Run 4+ (150+ FOVs) | Full 300 epochs. Majority-vote acceptance: accept any FOV where <5% ROIs need edits. |

NOTE  Agreement rate (fraction of model-proposed ROIs accepted without edits) is your primary signal. Targeting 95%+ agreement before moving to fully automated inference is a reasonable threshold for two-photon data.

SECTION 7 ¦ Batch Inference and Trace Extraction

# 7. Automated Batch Inference

## 7.1  Batch Inference Script

Once the model reaches your agreement threshold, deploy it for automated batch processing. Save as ~/roigbiv/scripts/run_inference.py:

#!/usr/bin/env python3 """ ROI G. Biv — Batch inference on new imaging sessions. Outputs: uint16 mask .tif and summary CSV per FOV. Usage: python run_inference.py --input_dir /path/to/session --diameter 12 """ import argparse, csv from pathlib import Path import numpy as np import tifffile from cellpose import models MODEL_PATH = Path.home() / 'roigbiv' / 'models' / 'deployed' / 'current_model' OUT_DIR = Path.home() / 'roigbiv' / 'inference' / 'output' OUT_DIR.mkdir(parents=True, exist_ok=True) def run_inference(input_dir, diameter): input_dir = Path(input_dir) model = models.CellposeModel(gpu=True, pretrained_model=str(MODEL_PATH)) results = [] for tif in sorted(input_dir.glob('*_mean.tif')): print(f'Processing {tif.name} ...') img = tifffile.imread(tif).astype(np.float32) masks, flows, styles = model.eval( img, diameter=diameter, channels=[0, 0], flow_threshold=0.4, cellprob_threshold=0.0, normalize=True, ) n_rois = masks.max() out_mask = OUT_DIR / tif.name.replace('_mean.tif', '_masks.tif') tifffile.imwrite(out_mask, masks.astype(np.uint16)) results.append({'fov': tif.stem, 'n_rois': n_rois, 'mask': str(out_mask)}) print(f' -> {n_rois} ROIs') # Write summary summary_path = OUT_DIR / 'inference_summary.csv' with open(summary_path, 'w', newline='') as f: w = csv.DictWriter(f, fieldnames=['fov', 'n_rois', 'mask']) w.writeheader() w.writerows(results) print(f'\nSummary: {summary_path}') if __name__ == '__main__': ap = argparse.ArgumentParser() ap.add_argument('--input_dir', required=True) ap.add_argument('--diameter', type=float, default=0, help='Expected cell diameter in pixels. 0 = auto-estimate.') args = ap.parse_args() run_inference(args.input_dir, args.diameter)

# Run on a new imaging session python ~/roigbiv/scripts/run_inference.py \ --input_dir /path/to/new/session \ --diameter 12

## 7.2  Fluorescence Trace Extraction

Once masks are generated, extract mean fluorescence traces per ROI from the raw stack. Save as ~/roigbiv/scripts/extract_traces.py:

#!/usr/bin/env python3 """ Extract mean fluorescence traces from raw stacks using Cellpose masks. Output: CSV and npy file per FOV, rows=frames, cols=ROI IDs. Usage: python extract_traces.py --raw_dir /path/to/raw --mask_dir /path/to/masks """ import argparse import numpy as np import tifffile import pandas as pd from pathlib import Path OUT_DIR = Path.home() / 'roigbiv' / 'inference' / 'traces' OUT_DIR.mkdir(parents=True, exist_ok=True) def extract_traces(raw_dir, mask_dir): raw_dir = Path(raw_dir) mask_dir = Path(mask_dir) for mask_path in sorted(mask_dir.glob('*_masks.tif')): stem = mask_path.stem.replace('_masks', '') # Try to find matching raw stack raw_candidates = list(raw_dir.glob(f'{stem}*.tif')) if not raw_candidates: print(f'WARNING: No raw stack for {stem}') continue raw_path = raw_candidates[0] print(f'Extracting traces: {stem}') stack = tifffile.imread(raw_path).astype(np.float32) mask = tifffile.imread(mask_path) if stack.ndim == 2: stack = stack[np.newaxis, ...] n_frames = stack.shape[0] roi_ids = np.unique(mask[mask > 0]) traces = np.zeros((n_frames, len(roi_ids)), dtype=np.float32) for i, roi_id in enumerate(roi_ids): roi_pixels = mask == roi_id for t in range(n_frames): traces[t, i] = stack[t][roi_pixels].mean() # Save as CSV and npy df = pd.DataFrame(traces, columns=[f'ROI_{r}' for r in roi_ids]) df.to_csv(OUT_DIR / f'{stem}_traces.csv', index=False) np.save(OUT_DIR / f'{stem}_traces.npy', traces) print(f' {len(roi_ids)} ROIs x {n_frames} frames') if __name__ == '__main__': ap = argparse.ArgumentParser() ap.add_argument('--raw_dir', required=True) ap.add_argument('--mask_dir', default=str(Path.home() / 'roigbiv/inference/output')) args = ap.parse_args() extract_traces(args.raw_dir, args.mask_dir)

SECTION 8 ¦ Troubleshooting

# 8. Troubleshooting

## 8.1  CUDA / GPU Issues

| Symptom | Fix |
| --- | --- |
| torch.cuda.is_available() returns False | Switch to NVIDIA mode in Pop!_OS settings and reboot. Verify with nvidia-smi. |
| CUDA out of memory during training | Reduce --batch_size to 2. If OOM persists at batch 2, your images may be larger than 512x512 — downsample in preprocessing. |
| GPU utilization <30% during training | CPU is the bottleneck. Check that image preprocessing (normalization) is not happening inside the training loop per-batch. |
| nvidia-smi shows GPU but Cellpose uses CPU | Ensure NVIDIA mode is active (not Hybrid). Run: sudo system76-power graphics nvidia then reboot. |

## 8.2  Training Issues

| Symptom | Fix |
| --- | --- |
| Loss NaN after epoch 1 | Learning rate too high. Reduce --lr to 0.01. Check for all-zero masks in dataset. |
| Val loss much higher than train loss | Overfitting. Add more annotated FOVs. Reduce epochs. Verify val split is representative. |
| Model predicts no ROIs on new data | Diameter mismatch. Run with diameter=0 (auto-estimate) first, then set explicit diameter once consistent. |
| ROIs massively over-segmented | Increase cellprob_threshold from 0.0 to 0.5-1.0 in inference parameters. |
| Training crashes mid-run | Check ~/roigbiv/logs/*.log for stack trace. Most common cause: a corrupted .tif file in dataset. Run validate_dataset.py. |

## 8.3  Data Issues

| Symptom | Fix |
| --- | --- |
| ROI conversion produces empty masks | Check that ImageJ ROI coordinates are in pixel units, not physical units. Re-export from Fiji with pixels selected. |
| Shape mismatch between image and mask | Projection script crops differently than Fiji's ROI coordinate space. Re-run make_projections.py without any cropping. |
| Only background in mean projection | Clipping percentiles too aggressive. Lower percentile_clip to (0.1, 99.9) in make_projections.py. |

## 8.4  Pop!_OS / Driver Issues

| Symptom | Fix |
| --- | --- |
| Black screen after switching to NVIDIA mode | Boot to recovery, switch back to hybrid: sudo system76-power graphics hybrid |
| Fans not responding during training | Install LenovoLegionLinux driver per Section 2.6. Check: ls /sys/class/hwmon/ |
| SSH session disconnects during overnight training | Use tmux or screen: tmux new -s train. Run training inside session. Detach with Ctrl+B D. |

SECTION 9 ¦ Quick Reference

# 9. Quick Reference

## 9.1  Daily Workflow Commands

# Activate environment conda activate roigbiv # Switch to NVIDIA mode (do once per session before training) sudo system76-power graphics nvidia # then reboot # Generate projections from new raw stacks python ~/roigbiv/scripts/make_projections.py ~/roigbiv/data/raw ~/roigbiv/data/annotated # Convert ImageJ ROIs to masks python ~/roigbiv/scripts/convert_rois.py ~/roigbiv/data/annotated ~/roigbiv/data/masks # Validate dataset python ~/roigbiv/scripts/validate_dataset.py # Train python ~/roigbiv/scripts/train.py --run_id runXXX --epochs 100 # Launch GUI for review python -m cellpose --gpu # Ingest corrections after GUI session python ~/roigbiv/scripts/ingest_corrections.py # Batch inference on new session python ~/roigbiv/scripts/run_inference.py --input_dir /path/to/session # Extract traces python ~/roigbiv/scripts/extract_traces.py --raw_dir /path/to/raw

## 9.2  Iteration Checklist

- Generate projections from raw stacks

- Convert or correct ROI annotations

- Validate dataset (validate_dataset.py)

- Train model (new run_id each time)

- Review predictions in Cellpose GUI

- Ingest corrections (ingest_corrections.py)

- Retrain with corrected dataset, base model = previous checkpoint

- Evaluate agreement rate — stop when consistently above 95%

- Promote model to deployed/current_model

- Run batch inference on new sessions

ROI G. Biv  —  The pipeline with a name you'll actually remember